| Key Features |  |  |
| --- | --- | --- |
|  |  | Optimized unrestricted flow Efficient on most sediments including rocks Compact design and standardized components Light weighted and very durable materials Easy to install and ROV friendly Backflush Hydraulic System Low Pressure Water jetting system with separate nozzle |

| Specifications |  |  |  |
| --- | --- | --- | --- |
| Approx. weight in air |  | 110kg |  |
| Approx. weight in water |  | 90kg |  |
| Max hydraulic Flow |  | 70 lpm |  |
| Max hydraulic Pressure |  | 200 bar |  |
| Flow rate and removal capacity (theoretical) |  | Water flow at pump: Gross water flow: Removal capacity sand: Removal capacity rocks: Max rock size: | 1400 lpm 8108 lpm 84 T/hr 41 T/hr 140mm |
| Hydraulic Interface |  | Pressure: Return: Drain: Backflush Close: Backflush Open: | ¾ Hose / JIC 12 ¾ Hose / JIC 12 ½ Hose / JIC 8 ¼ Hose / JIC 4 ¼ Hose / JIC 4 |